Monocular Visual Odometry

Monocular Visual Odometry (MVO) is a process of determining the position and orientation (pose) of a moving system by analyzing a set of images using a single camera. The goal is the estimate the motion of the system by determining how the system moves in each image frame. In this process, two consecutive images It and It+1 which are captured by the camera is selected to calculate the motion. These two images contain overlapping visual information from slightly different viewpoints due to camera motion. From these images, feature points like edges and corners of the frames are detected in It and tracked into It+1. Using the matched features, the Essential Matrix is estimated which encodes the relative rotation and translation between the two frames. From this, rotation matrix R and translation vector t are calculated. Lastly, the system’s trajectory is derived by accumulating these transformations over time.

Trajectory Plotting

Motion Estimation

Feature Matching

Feature Detection

Image Sequence

So, the flow chart of the basic principle of MVO is:

To implement MVO through a webcam, the implementation of 6 modules are required.

- Image Acquisition Module

- Feature Detection Module

- Feature Tracking Module

- Motion Estimation Module

- Pose Accumulation Module

- Inertial Measurement Unit (IMU) Module

- Trajectory Plotting Module

Image Acquisition Module

At first, image data is required to perform feature detection and tracking. For that, a webcam is implemented by using the OpenCV’s “VideoCapture” API, which enables real-time frame acquisition from the camera device. Using this, the webcam provides frame by frame image stream. Each frame is returned as a numerical array in BGR format for each time step. These captured frames serve as the input data for feature detection, tracking, and motion estimation.

Feature Detection Module

Feature detection is one the fundamental parts of MVO to estimate the motion and plot the trajectory of the moving system. The main purpose of feature detection is the detect strong pixels which will be used to track the motion in every frame. There are many algorithms which can detect the feature. But one of the most efficient algorithms for feature detection is “Shi-Tomasi Corner Detection Algorithm”. Shi-Tomasi detects strong corners in an image where intensity changes significantly in multiple directions. Mathematically, Shi-Tomsai computes the partial derivatives of intensity in a small window around each pixel. These represent intensity change in certain directions. Then, for each pixel, it computes, M =

(Ix)2IxIyIxIy(Iy)2

This matrix describes how intensity varies around the pixel. Based on this, the algorithm computes eigenvalues (λ1, λ2) which represent intensity variation in two perpendicular directions. It uses a rule where, R (corner response value) = min (λ1, λ2). If R > threshold then the point is considered a good corner meaning a good feature. The output given by Shi-Tomasi algorithm is an array of 2D pixel co-ordinates which represent detected feature location.

Feature Tracking Module

After detecting feature points in a frame, it is essential to find those same physical points moved in next frame. Feature tracking module takes detected feature from one frame and estimates the new positions in the next frame. For this, one of the most used methods in “Lucas–Kanade Optical Flow algorithm”. This method assumes that the intensity of a moving point remains constant between frames. Meaning,

I(x,y,t)=I(x+dx,y+dy,t+1)

Using this constraint, the algorithm computes the pixel motion within a local window. To ensure the utmost reliability, a forward backward point consistency calculation can be useful to remove unstable points.

Motion Estimation Module

Motion estimation module is where matched feature points between two frames are taken to compute the relative orientation and position of the system. It calculates how the camera moved from one frame to another. The motion estimation is done using Nistér’s 5-Point Algorithm and RANSAC for outlier rejection. The foundation of motion estimation in MVO is epipolar geometry. In motion estimation, the epipolar constraint is used to find the essential matrix.

X2T​E(X1)​=0
Where,

X1​ = normalized point in frame 1

X2​ = normalized point in frame 2

E = Essential Matrix

Here, Essential matrix is a 3X3 matrix to describe the relative rotation and translation between two camera views.

E=[t]TR

Where,

R = 3x3 rotation matrix which describes the change in orientation

t = 3x1 translation vector which describes the direction of motion

[t]T = skew symmetric matrix of t

After estimating E, it is decomposed to get the Rotation matrix R and Translation Vector t.

Pose Accumulation Module

Pose accumulation is the core of MVO, as it does the estimation of the trajectory of the system using the frames from moving camera. This module computes the relative motion between consecutive frames, and integrates these motions over time to reconstruct the overall path of the system. The relative transformations are assembled sequentially to calculate the global pose. If Pk​ represents the camera position at frame k, and ΔPk denotes the relative displacement between frame k−1 and frame k then he global position is updated as:

Pk = Pk-1 + ΔPk

MVO does not directly provide a scale to estimate proper magnitude of displacement. For this reason, an Inertial Measurement Unit (IMU) is used to estimate the magnitude of displacement. Thus, the pose accumulation module converts relative motion into a global trajectory representation, forming the final output of the visual odometry system.

Inertial Measurement Unit (IMU) Module

Inertial Measurement Unit works to provide motion related measurement that complement the visual information in MVO system. An IMU sensor uses gyroscope and accelerometers to measure rotational motion and linear acceleration, which enables the estimation of rotational matrix and translational vector. The primary outputs of IMU that helps the MVO system are: 1. Orientation, 2. Linear Acceleration. The orientation output is used to calculate the rotation matrix and the velocity magnitude is used to determine the translation of the system.

Trajectory Plotting Module

Trajectory plotting module helps to visualize the estimated motion accumulated from the camera frames in a two-dimensional plane. After pose accumulation module computes the global position at each frame, this module provides the graphical presentation of the system’s path over time. The accumulated positions are stored as a sequence of three-dimensional coordinates:

Pk = (xk, yk, zk)

In MVO, most motion occurs along the horizontal plane. So, the trajectory is typically illustrated using the X and Z components of the position.